Page 1 of 1

| Corporate Finance Department Purchasing Division | 732-2024 ADDENDUM 1 |  |
| --- | --- | --- |
| SUPPLY AND DELIVERY OF GROUND MATS FOR THE BRADY ROAD RESOURCE MANAGEMENT FACILITY |  |  |
|  |  | ISSUED: October 15, 2024 |
|  |  | BY: Sean Steeves |
| URGENT PLEASE FORWARD THIS DOCUMENT TO WHOEVER IS IN POSSESSION OF THE BID/PROPOSAL |  | TELEPHONE NO. 204 986-2000 |
|  |  | THIS ADDENDUM SHALL BE INCORPORATED INTO THE BID/PROPOSAL AND SHALL FORM A PART OF THE CONTRACT DOCUMENTS Template Version: Add 2024-02-01 |
| Please note the following and attached changes, corrections, additions, deletions, information and/or instructions in connection with the Bid/Proposal, and be governed accordingly. Failure to acknowledge receipt of this Addendum in Paragraph 10 of Form A: Bid/Proposal may render your Bid/Proposal non-responsive. |  |  |

PART E – SPECIFICATIONS

Add:	E2.3: Specifications of Ground Mats E2.3.1	Dimensions and Material:

Ground mats must be constructed from high-density polyethylene (HDPE) or an equivalent material.

Each mat should have minimum dimensions of 2.44 meters by 4.27 meters (8 feet by 14 feet) and a thickness of 50 millimetres (2 inches).

E2.3.2	Load Capacity:

Ground mats must be rated for a load capacity of at least 80 tonnes, suitable for machinery use in landfill environments.

E2.3.3	Interlocking System:

Mats must feature an interlocking system to ensure secure connections under heavy loads and prevent displacement during use.

E2.3.4	Environmental Resistance:

The mats must be resistant to UV exposure, chemicals, and general environmental degradation, ensuring durability for long-term outdoor use.

E2.3.5	Recyclability:

Ground mats should be made from recyclable materials and have the potential for recycling at the end of their lifecycle, aligning with environmental sustainability goals.